November 16, 2016, TMT Meeting
Potential Modification to the Current Chum Operation
1. EFFECTIVE TUESDAY, NOVEMBER 1, AT 0600 HOURS, OPERATE THE 

BONNEVILLE DAM TAILWATER IN THE FOLLOWING ORDER OF OPERATING 

RANGES AS PROJECT OUTFLOW INCREASES. THE INTENT OF THIS 

OPERATION IS TO IMPROVE CONDITIONS AT THE IVES/PIERCE ISLAND 

COMPLEX FOR CHUM SALMON SPAWNING. 

2. DURING ALL HOURS, OPERATE PROJECT OUTFLOW TO PROVIDE A 

TAILWATER ELEVATION IN THE RANGE OF 11.5-13.0 FEET.  

3. THEN, IF NECESSARY TO INCREASE PROJECT OUTFLOW, THE 

TAILWATER MAY BE OPERATED UP TO 16.5 FEET DURING NIGHTTIME 

HOURS (1700-0600). CONCENTRATE HIGHEST ELEVATIONS AROUND 

2400 HOURS.

4. THEN, IF NECESSARY TO INCREASE PROJECT OUTFLOW, THE 

TAILWATER MAY BE OPERATED UP TO 16.5 FEET ALL HOURS.  
5. THEN, IF INCREASING RIVER FLOW PRECLUDES THE ABILITY TO 

MANAGE TAILWATER WITHIN THE STEPS ABOVE, OPERATE TO PROVIDE 

A TAILWATER UP TO 16.5 FEET DURING DAYTIME HOURS (0600-1700) AND 
UP TO 18.5 FEET DURING NIGHTTIME HOURS (1700-0600).

6.  THEN, IF NECESSARY TO INCREASE PROJECT OUTFLOW, THE 

TAILWATER MAY BE OPERATED UP TO 18.5 FEET ALL HOURS.  
7. THEN, IF INCREASING RIVER FLOW PRECLUDES THE ABILITY TO 

MANAGE TAILWATER WITHIN THE STEPS ABOVE, OPERATE TO PROVIDE 

A TAILWATER IN THE RANGE OF 18.5 FEET DURING DAYTIME 

HOURS (0600-1700) AND UP TO THE MAXIMUM WITHIN PROJECT 

24-HOUR RAMP RATE LIMITS FEET DURING NIGHTTIME HOURS (1700-0600).

