October 25, 2017 TMT Meeting
TMT Coordinated Chum Operation

1. EFFECTIVE NO LATER THAN TUESDAY, NOVEMBER 7, AT 0600 HOURS, OPERATE THE 

BONNEVILLE DAM TAILWATER IN THE FOLLOWING ORDER OF OPERATING 

RANGES AS PROJECT OUTFLOW INCREASES. 

2. DURING ALL HOURS, OPERATE PROJECT OUTFLOW TO PROVIDE A 

TAILWATER ELEVATION IN THE RANGE OF 11.5-13.0 FEET.  

3. THEN, IF NECESSARY TO INCREASE PROJECT OUTFLOW, THE 

TAILWATER MAY BE OPERATED UP TO 16.5 FEET DURING NIGHTTIME 

HOURS (1700-0600). CONCENTRATE HIGHEST ELEVATIONS AROUND 

2400 HOURS.

4. THEN, IF NECESSARY TO INCREASE PROJECT OUTFLOW, THE 

TAILWATER MAY BE OPERATED UP TO 18.5 FEET DURING NIGHTTIME 

HOURS (1700-0600).  

5. THEN, IF INCREASING RIVER FLOW PRECLUDES THE ABILITY TO 

MANAGE TAILWATER WITHIN THE STEPS ABOVE, OPERATE TO PROVIDE 

A TAILWATER IN THE RANGE OF 13.0-16.5 FEET DURING DAYTIME 

HOURS (0600-1700) AND UP TO THE MAXIMUM WITHIN PROJECT 

24-HOUR RAMP RATE LIMITS DURING NIGHTTIME HOURS (1700-0600).

