# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title: 	17BON002 – Kelt Trigger Wording Clarification

Date Submitted:		June 9, 2016

Project:			BON

Requester Name, Agency:	Nathan Zorich, COE FFU, and Gary Fredricks, NOAA Fisheries

Final Action:			APPROVED – 7/14/16

FPP Section:  BON 2.3.2.5. Juvenile Facilities - Fish Passage Season (Mar 1-Nov 30) – PH2

Justification for Change:  Current kelt trigger wording is confusing.

Proposed Change:

Current language:

2.3.2.5.t. Powerhouse Two Corner Collector (B2CC).  Operate the B2CC during fish passage spill season, April 10-August 31.  Open and close the B2CC within 1 hour of the start and end of spill season, respectively.  During spill that occurs September 1–April 9 (outside of fish passage spill season), the B2CC will be operated if available to provide a surface passage route.

t.1. Beginning March 1, JMF personnel will enumerate steelhead kelt at the JMF adult/debris separator.  If 2 kelts per day are observed at the JMF separators for 2 consecutive days for a cumulative total of 20 kelts, JMF personnel will notify the Control Room and Project Fisheries, and the B2CC will be opened within 1 hour.

Proposed language:

2.3.2.5.t. Powerhouse Two Corner Collector (B2CC).

t.1. March 1-April 10 (kelt trigger): To facilitate downstream migration of steelhead kelts, the B2CC will be opened if the following criteria are achieved.  Kelt will be counted at the JMF adult/debris separator starting March 1. If 2 kelts per day are counted on 2 consecutive days, and the total cumulative count since March 1 is greater than or equal to 20 kelts, JMF personnel will notify Project Fisheries and the Control Room, and the B2CC will be opened within 1 hour.

t.2. April 10-August 31 (Spring & Summer Spill): The B2CC will be open during spring and summer spill for fish passage, April 10-August 31, as defined in the FOP (Appendix E).  The B2CC will be opened within one hour of the start of spring spill on April 10 (unless already open for kelt, as defined above in t.1) and closed within one hour of the end of summer spill on August 31.

t.3. September 1-April 9: During spill that occurs outside of fish passage spill season, the B2CC will be opened if available to provide a surface passage route.

Comments:

Record of Final Action:  	APPROVED – 7/14/16