# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	18BON005 – Unit Priority Change

Date Submitted: 		May 22, 2018

Project: 			Bonneville Dam

Requester Name, Agency: 	Andrew Derugin, USACE BON

Final Action:			APPROVED 6/14/18

FPP Section: Table BON-13. Unit Priority Order.

Justification for Change:  Unit 9 is currently first off/last on in priority at PH1.  We would like to switch it with unit 2, which is currently 5th.  Unit 9 is important to have running so that chiller water (air condition unit) discharge can be released to the gate well of a running unit when unit 8 is not operating, to avoid fish kills.  There are no known warm water discharges into the unit 2 gate well.

When the air conditioning unit for PH1 is turned on, it discharges warm water into one of two tail log gate wells, 8A or 9B.  This water is warm, between 74 and 85* F.  When this water dumps into a gate well and the unit is not running, there is no mixing, and the water in the gate well is heated.  The gate wells of non-running units tend to hold fish, and the temperature stress likely kills them.

Proposed Changes:

Table BON-13. Bonneville Dam Turbine Unit Priority Order.

| PERIOD / OPERATION | UNIT PRIORITY ORDER * |
| --- | --- |
| Year-Round: Fish Ladders in service; PH1 Ladder out of service | PH2: 11, 18, 12, 17, 13, 14, 15, 16, Then PH1: 1, 10, 3, 6, 9, 4, 5, 8, 7, 2 |
| PH2 Fish Ladder out of service | PH1: 1, 10, 3, 6, 9, 4, 5, 8, 7, 2 Then PH2: 11, 18, 12, 17, 13, 14, 15, 16 |
| Split Flows (section 2.1.3) | PH1: 1, 10 (or top two available PH1 priority units), Then PH2: 11, 18, 12, 17, 13, 14, 15, 16, Then PH1: 3, 6, 9, 4, 5, 8, 7, 2 |
| PH1 Unit Priority | 1, 10, 3, 6, 9, 4, 5, 8, 7, 2 |
| PH2 Unit Priority | 11, 18, 12, 17, 13, 14, 15, 16 |

Comments: (listed from oldest to newest)

6/14/18 FPOM Meeting: Conder said that there could be an issue with the sluice way opening and whether the tailrace egress could change but overall, it didn’t seem too impactful. FPOM concurred.

Record of Final Action:  Approved at FPOM 6/14/18