# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	18LMN001 – Table LMN-11 modification

Date Submitted: 		8/25/17

Project: 			Lower Monumental

Requester Name, Agency: 	Ann Setter, USACE

Final Action:			APPROVED 9/14/17

FPP Section:  Table LMN-11 Spill Patterns with No RSW

Justification for Change: Need an additional row to cover a higher spill level.

Proposed Change:

Table LMN-11. Lower Monumental Dam Spill Patterns with No RSW (Bay 8 Closed).

| LMN Spill Patterns w/ NO RSW - # Gate Stops per Spillbay |  |  |  |  |  |  |  | Total Stops | Spill |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bay 1 | Bay 2 | Bay 3 | Bay 4 | Bay 5 | Bay 6 | Bay 7 | Bay 8 b | (#) | (kcfs) |
|  | 1 |  |  |  |  |  | CLOSE | 1 | 1.8 |
|  | 1 |  |  |  |  | 1 | CLOSE | 2 | 3.6 |
|  | 1 |  |  | 1 |  | 1 | CLOSE | 3 | 5.4 |
|  | 2 |  |  | 1 |  | 1 | CLOSE | 4 | 6.9 |
|  | 2 |  |  | 1 |  | 2 | CLOSE | 5 | 8.4 |
|  | 2 |  |  | 2 |  | 2 | CLOSE | 6 | 9.9 |
|  | 2 |  |  | 2 |  | 3 | CLOSE | 7 | 11.4 |
|  | 2 |  |  | 3 |  | 3 | CLOSE | 8 | 12.9 |
|  | 2 |  |  | 3 |  | 4 | CLOSE | 9 | 14.3 |
|  | 2 |  |  | 3 |  | 5 | CLOSE | 10 | 16.0 |
|  | 2 |  |  | 4 |  | 5 | CLOSE | 11 | 17.4 |

Comments:

9/14/17 FPOM:  FPOM is ok with adding one new row to the “No RSW” spill pattern table for higher spill of 17.4 kcfs.

Conder noted that the existing patterns with one stop in Bay 5 (total spill of 5.4–8.4 kcfs) might be better if that spill is shifted to Bay 7. It would be great to look at these patterns at ERDC next week (Sept 18-22) if possible, then revise if necessary.

Record of Final Action:  APPROVED at FPOM 9/14/17.