# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	19AppD001 – TDA Updates

Date Submitted:		11/29/2018

Project:			TDA

Requester Name, Agency:	Erin Kovalchuk, USACE

Final Action:			APPROVED – 2/7/2019

FPP Section:  Appendix D – Operations for Non-Listed Species, section 3.1. (TDA)

Justification for Change:

Weir caps were added to the east ladder entrance weirs this winter and last winter.

Proposed Change: [see below with edits to existing FPP in track changes]

- 3. The Dalles Dam.

- 3.1. Adult Lamprey Facilities.

- 3.1.1. Passage improvements were made in the east fish ladder by installing four orifice ramps to eliminate 90° edges. Several concrete 90° edges were also rounded with 2” radius. Picket leads were raised 1.5” for both north and east count stations. Steel plates for lamprey attachment substrate were installed in the lower 14 weir orifices. Weir caps have been added to all entrance weirs on both the east and north ladder.

Comments:

Record of Final Action:  Approved at the FPOM FPP meeting on 2/7/2019.