# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	19AppD003 – NWW Updates

Date Submitted:		1/31/2019

Project:			Walla Walla District Projects

Requester Name, Agency:	Ken Fone, Ice Harbor, USACE

Final Action:			APPROVED – 2/7/2019

FPP Section:  Appendix D – Operations for Non-Listed Species, section 5 (NWW)

Justification for Change:  This section has not been updated for many years.  This change form reorganizes and consolidates information, corrects some errors, and adds missing information.

Proposed Change: [see following pages for edits to existing FPP in track changes]

Comments:

2/8/19 Charles Morrill, WDFW, via email: “Ensure weekly project reports provide update on facility impacts to lamprey and or early migrating subyearling chinook.”

Record of Final Action:  Approved at the FPOM FPP meeting on 2/7/2019.

- NWW PROJECTS

  - Improvements for Lamprey.

    - At all projects, horizontal slots were cut at the bottom of the stem walls in the upper section of the ladders to allow adult lamprey attachment along a level pathway through the weirs.  Picketed leads were raised and secured 1.5” off the ladder floor at the count stations to enable adult lamprey passage through a low-velocity passage route under the picketed leads around the adult fish count slot.

    - In the McNary (Oregon shore), Ice Harbor, Little Goose, and Lower Granite fish ladders, plating was attached on the diffuser gratings adjacent to the submerged orifices just above tailwater.

    - At Ice Harbor and Lower Monumental dams, ramps were installed in some of the upper ladder weirs from the ladder floor to the bottom of elevated salmon orifices to assist lamprey in maintaining attachment as they maneuver through these area.

    - Lamprey passage structures were installed at one of the south shore entrances of both the Oregon fish ladder at McNary and the south shore fish ladder at Ice Harbor.

    - Lamprey-friendly raceway tailscreens were installed at all of the juvenile fish facilities that collect fish for transportation (Lower Granite, Little Goose, and Lower Monumental).  These tailscreens allow collected juvenile lamprey to volitionally pass through the mesh and return to the river rather than be transported. At Lower Granite and Little Goose, the tailscreen wire mesh diameter is 1.6 mm (0.063”) with an open width/height of 8.6 mm (0.337”), open diagonal dimension of 12.1 mm (0.477”) and overall screen open area of 71.0%. Lower Monumental changed to a perforated plate tailscreen that can be cleaned with brushes without entangling lamprey. The plate is 1/4” thick with 0.312” x 1.0” slots spaced 1/4” apart oriented vertically in a side-staggered pattern.

    - At McNary, the overflow screens in the sample and PIT-tag holding tanks were changed from slotted to round-hole perforated plates.  The plates are 1/8” thick with 1/8” staggered holes that are small enough so that lamprey do not get caught in them.

    - When the adult and juvenile fish facilities are unwatered for annual or periodic maintenance, all projects perform routine maintenance on lamprey passageways.

  - Operations for Lamprey.

    - At McNary Dam, unit trash racks will be raked during the winter maintenance period prior to January 16 to minimize the potential for lamprey entanglement in built-up debris when river flow increases. See FPP Chapter 7 (MCN), section 2.3.1.1.

    - At McNary Dam, ESBSs will be installed and operating between April 2 and April 15 (about two weeks later than other NWW projects) to allow juvenile lamprey passage directly through turbines without bypass collection. See FPP Chapter 7 (MCN), section 2.3.2.2.

    - At Ice Harbor, Lower Monumental, Little Goose, and Lower Granite, fallback adult lamprey collected off fish separators and other areas of the juvenile fish facilities will be released into the forebay rather than being bypassed back into the tailrace or transported downstream.

    - Turbine cooling water strainer inspections will be conducted once per month from mid-December until mid-June at the four lower Snake River dams, and from mid-December to mid-July at McNary Dam. If 10 or more juvenile lamprey are collected during the last sample date in June/July, an additional month of inspections should be made.