# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:  	19IHR003 – Operation of Fish Ladder Entrance

Date Submitted: 		December 19, 2018

Project: 			Ice Harbor

Requester Name, Agency: 	Ken Fone, Ice Harbor Dam, USACE

Final Action:			APPROVED – 2/7/2019

FPP Section:  Ice Harbor 2.4.2.4 and 2.4.2.7 (Adult Facilities, Fish Passage Season)

Justification for Change:  This change form provides clarification that there are two entrances at each location, and clarifies when NEW-2 and SFE-2 will be operated in place of NEW-1 and SFE-1, respectively.

Proposed Change: [see below with edits to existing FPP in track changes]

- 2.4.2.4. North Shore Entrances (NEW-1 and NEW-2). Elevation at top of gate on sill = 332.25’.

            - Operate downstream gate closest to shore (NEW-1).

            - NEW-2 will only be operated for adult salmonid passage in the event that NEW-1 gate is not functioning properly to maintain criteria.  NEW-1 gate shall be repaired and returned to service as soon as possible

            - Weir depth 8' or greater below tailwater. At tailwaters less than 340.25', weirs should be on sill. Note that at low river flow and tailwater, some of the diffusers are above tailwater and project may only be able to maintain a 6' weir depth.

            - North Shore Lower Diffuser Gates: If tailwater is below 344', diffuser gates should be fully open. If tailwater is above 344', diffuser gates should be ½ open.

- 2.4.2.5. North Powerhouse Entrances (NFE-1&2). Elevation at top of gate on sill=332.25'.

            - Operate one downstream gate.

            - Weir depth 8' or greater below tailwater. At tailwaters less than elevation 340.25', weirs should be on sill. [Note: At low tailwater, weirs will bottom out and will be less than 8' below tailwater.]

- 2.4.2.6. Floating Orifice Gates. Operate 4 floating orifices, OG1, 4, 10, and 12.

- 2.4.2.7. South Shore Entrances (SFE-1 and SFE-2). Elevation of top of SFE-1 gate when on sill = 332.25'. Elevation of top of SFE-2 gate when on sill = 333.25’ with the installation of the lamprey passage structure.

            - Operate entrance closest to powerhouse (SFE-1).

            - SFE-2 will only be operated for adult salmonid passage in the event that SFE-1 gate is not functioning properly to maintain criteria.  SFE-1 gate shall be repaired and returned to service as soon as possible.

            - Weir depth: 8' or greater below tailwater. At tailwaters less than elevation 340.25' for SFE-1 and 341.25’ for SFE-2, weirs should be on sill. [Note: At low tailwater, weirs will bottom out and will be less than 8' below tailwater.]

Comments:

2/7/19 FPP Meeting: Van Dyke and Morrill requested that these entrances be added to the figure in the FPP (Figure IHR-1). Wright will do that.

Record of Final Action:  Approved at the FPOM FPP meeting on 2/7/2019. Wright will add entrances NEW-2 and SFE-2 to the FPP Figure IHR-1.