# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title: 	19LGS003 - Little Goose Dam Fishway Cooling Pump Criteria

Date Submitted:		December 20, 2018

Project:			Little Goose Dam

Requester Name, Agency:	Chris Peery, USACE NWW

Final Action:			APPROVED – 2/7/2019

FPP Section:  2.4.2.14. Adult Fish Facilities – Fish Passage Season (March 1 – Dec 31) - Ladder Exit Pool Cooling Pumps.

Justification for Change:  To simplify and clarify operations criteria for fishway cooling pump and make criteria consistent between Little Goose and Lower Granite dams.

Proposed Change: (edits to existing FPP text in track changes)

2.4.2.14. Adult Fish Ladder Exit Pool Cooling Pump. Operate forebay exit pool cooling pump that sprays upstream of the fish ladder exit to enhance conditions for adult fish exiting the ladder and to supplement cooler water throughout the ladder. The water supply for the manifold at the exit pool originates from an added forebay pump with intake at elevation 543’ in the forebay, which is 90’ below the MOP range (633-638’).

i. Begin operation of exit pool cooling pump after June 1 and no later than the day following when the Little Goose forebay temperature string at 0.5 meters exceeds 64°F (18°C) at any time.  Forebay temperature string data are online at: http://pweb.crohms.org/ftppub/water_quality/tempstrings

ii. Continue this operation until September 1 and until the Little Goose forebay temperature string at the 0.5 m  is less than 68°F (20°C) for 3 consecutive days. Restart pumps if 0.5 m temperatures reach 68°F (20°C) at any time and follow above criteria on when to discontinue pump operation.

iii. The pump may be turned on or off at the Project Biologist’s discretion if adult passage delays are observed either in the forebay or within the ladder, and operation of the pump is believed to influence the adult passage issue.

Comments:

Record of Final Action:  	Approved at the FPOM FPP meeting on 2/7/2019.