# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title: 	21JDA001 – Dates for Spillbay 2 Attraction Flow

Date Submitted:		December 11, 2020

Project:			John Day Dam

Requester Name, Agency:	Lisa Wright, Corps RCC, and Eric Grosvenor, Corps JDA

Final Action:			APPROVED – January 28, 2021

FPP Section:

JDA sections 2.2 (Spill Mgmt) and 2.4.2 (Adult Facilities/Passage Season)

Justification for Change:

As coordinated in-season during 2020 summer spill via MOC 20JDA12, this change form adds the operation of spillbay 2 for adult attraction during daylight hours August 15-31 when spill is through the TSWs only. This operation maintains attraction flow to the north end of the spillway near the North fish ladder entrance.

Proposed Change:

- 2.2. 	Spill Management

2.2.3. From August 15 through November 30, adult fish attraction flow will be provided by spilling through Bay 2 open one stop (approximately 1.6 kcfs) during daylight hours defined in Table JDA-5.

2.4.2.	Adult Fish Facilities – Adult Fish Passage Season (March 1 – November 30)

- 2.4.2.9. South Fishway. Operate entrance weir SE-1. Provide adult attraction flow from August 15 through November 30 by spilling from Bay 2 open one stop (approximately 1.6 kcfs) during daylight hours defined in Table JDA-5.

Comments:

Record of Final Action:  Approved at the FPOM FPP meeting on 28-JAN-2021.